## TZONE.RD07.dll description

Support RD07、RD07-WiFi、RD07-4G

- Data analy

- 一、Ordinary data protocols

| Namespaces | Type name | Type name |
| --- | --- | --- |
| TZONE.RD07.Protocols | Generic | Analyse ordinary data |

- Class attribute description

| Name | Type | Specification |
| --- | --- | --- |
| HardwareType | string | Hardware type |
| FirmwareVersion | string | Firmware version |
| IMEI | string | IMEI |
| RTC | DateTime | Package time of terminal data（Note：1970 means null） |
| LACCI | string | LBS information(mcc: mnc: LAC:CID:) |
| Alram | string | Alarm type |
| TerminalInfo | string | Terminal information |
| GSMCSQ | int | GSM signal strength |
| GSMStatus | string | GSM status |
| VBV | decimal | Battery voltage |
| VCV | Decimal | Recharging voltage |
| Tag0708List | Tag | Tag0708 |
| Tag07CList | Tag | Tag07C |
| Serial | string | Serial number |
| ServerTime | DateTime | Server Time |
| Length | int | Package length |
| Message | string | Data |

- Class method

Analyse data：Generic Analysis(byte[] data);

- 二、Tags data protocols

| Namespace | Type name | function |
| --- | --- | --- |
| TZONE.RD07.Protocols | TAG | Analyse tags data |

- Class attribute description

| Name | Type | Specifications |
| --- | --- | --- |
| SN | string | Tag number |
| Temperature | decimal | Temperature value(Note：when the value is -1000, means Null) |
| Humidity | decimal | Humidity value(Note：when the value is -1000, means Null) |
| Pressure | decimal | Pressure value(Note：when the value is -1000, means Null) |
| VBV | decimal | voltage(Note：when the value is -1000, means Null) |
| V | decimal | Voltage (Note: value = -1000, which means empty) |
| A | decimal | Current (Note: value = -1000, which means empty) |
| Switch | Int | Switch quantity (Note: 1 is on and 0 is off) |
| RSSI | int | Signal strength |
| IsLowVoltage | Bool | If it is low voltage |
| RTC | DateTime | Package time of terminal data（Note：1970 means null） |

Others

Device RTC time setting：

Format：@UTC,{0}#

For example: @UTC, 2017-10-20 08:01:01#

ACK:

Format：@ACK, serial#

For example: @ACK, 100#