## TZONE.TT19.dll Instruction

- Data Analysis

- Data analysis

| Name | Class | Function |
| --- | --- | --- |
| TZONE.Hardware.TT19. TcpUdp | DD | Decode |

- DataPacket Attribute

| Name | Type | Description |
| --- | --- | --- |
| FirmwareVersion | string | Firmware Version |
| HardwareType | string | Hardware Type |
| IMEI | string | IMEI |
| RTC | DateTime | Terminal data packing time |
| TrackerTime | DateTime | Gps Time |
| Lat | Double | latitude |
| Lng | Double | longitude |
| Direction | Double | Direction |
| Speed | Double | Speed |
| NetworkType | String | Network Type |
| LACCI | String[] | LACCI |
| Temperature1 | Double | Built-in temperature |
| Humidity1 | Double | Built-in humidity |
| Temperature2 | Double | External temperature |
| Humidity2 | Double | external humidity |
| Light | Int | Light |
| Vibration | Double | Vibration |
| WiFi | String[] | MAC address of nearby WiFi |
| LocationSource | String | Location Source |
| Alram | string | Alram Type |
| TerminalInfo | string | End point information |
| GSMCSQ | int | GSM signal strength |
| GSMStatus | string | GSM status |
| VBV | Double | battery voltage |
| Battery | Double | Battery level |
| Serial | string | Serial |
| ServerTime | DateTime | Server Time |
| Length | int | Data packet length |
| Message | string | Raw data |

- Class Method

- DecodePackage：DataPacket DecodePackage (byte[] rawMessage);

Example：

545A006C2424040A01020000019002300000000617080E071F0000130117080E071F140157E33C06CC8DC400000000001401D20460000027B60A7E740D0059009600600843000EAA001737003A010F022F000D0000001508570072D1804B18D98F1618EC3D7405A5E3E03324000DE5A00D0A

=>

{"TT19":{"LACCI":[["460","000","27B60A7E740D"]],"WiFi":[["08:57:00:72:D1:80","-75"],["18:D9:8F:16:18:EC","-61"],["74:05:A5:E3:E0:33","-36"]],"IMEI":"190023000000006","HardwareType":"040A","FirmwareVersion":"01.02.00.00","RTC":"2023-08-14T07:31:00","TrackerTime":"2023-08-14T07:31:20","Lat":22.53702,"NS":"N","Lng":114.068932,"WE":"E","Direction":0.0,"Speed":0.0,"NetworkType":"LTE","Alram":"AA","TerminalInfo":"00000000","GSMCSQ":23,"GSMStatus":"00110111","VBV":"NaN","Battery":58.0,"Temperature":27.1,"Temperature2":"NaN","Humidity":0.55899999999999994,"Humidity2":"NaN","Light":13.0,"Vibration":0.0,"LocationSource":"LACCI","ServerTime":"2023-08-14T09:28:07.4067199Z","Message":"545A006C2424040A01020000019002300000000617080E071F0000130117080E071F140157E33C06CC8DC400000000001401D20460000027B60A7E740D0059009600600843000EAA001737003A010F022F000D0000001508570072D1804B18D98F1618EC3D7405A5E3E03324000DE5A00D0A","Serial":"13"}}