## RD06(Transparent) API

Support RD06、RD06-WiFi、RD06-4G

- 一、Data Protocol

| Namespace | Class | Function |
| --- | --- | --- |
| TZONE.Hardware.RD06. { transport protocol type} | Decoder | Standard data packet decoder |
|  | DecoderFromTTP | Transparent Transmission protocol data packet decoder |

- Class Attribute

| Name | Type | Description |
| --- | --- | --- |
| HardwareType | String | Hardware Type |
| FirmwareVersion | String | Firmware Version |
| IMEI | String | IMEI |
| RTC | DateTime | Package time of terminal data（Note：1970 means null） |
| DataTypeID | String | Data Type ID |
| TerminalInfo | String | Terminal Info |
| GSMCSQ | Int | GSM signal strength |
| GSMStatus | String | GSM status |
| VBV | Decimal | battery Voltage |
| VCV | Decimal | Charging Voltage |
| Serial | String | Serial |
| ServerTime | DateTime | Server Time |
| Length | Int | Package Length |
| Message | String | Raw data |

- Method

Decode (byte[] rawMessage);

- 二、Tag Data Protocol

- Class Attribute

| Name | Type | Description |
| --- | --- | --- |
| SN | String | Tag Number |
| Temperature | Decimal | Temperature value(Note：when the value is -1000, means Null) |
| Humidity | Decimal | Humidity value(Note：when the value is -1000, means Null) |
| Pressure | Decimal | Pressure value(Note：when the value is -1000, means Null) |
| VBV | Decimal | voltage(Note：when the value is -1000, means Null) |
| RSSI | Int | Signal strength |
| IsLowVoltage | Bool | If it is low voltage |
| RTC | DateTime | Package time of terminal data（Note：1970 means null） |

- Other

Set Datetime：

Format：@UTC,{0}#

For example: @UTC, 2017-10-20 08:01:01#

ACK:

Format：@ACK, serial#

For example: @ACK, 100#